THE PHYSICS QUESTION BANK · AQA GCSE PHYSICS

# Power the Island – Energy Resources

A one-hour lesson. You will find out where our electricity comes from, decide which energy resources are best, and practise real exam questions.

Name: ______________________________ Class: ____________ Date: ____________

How to use this worksheet: work through the steps in order and write your answers in your exercise book. Copy every blue 📝 box neatly – these are your revision notes. When you have finished a step, check your work against the Answers at the end and correct anything in a different colour.

📝 COPY INTO YOUR BOOK By the end of this lesson I will be able to: 1. name the main energy resources and say which are renewable 2. compare them for reliability and their effect on the environment 3. answer a "compare" and an "evaluate" exam question.

## Step 1: Starter (5 minutes)

Think about the last time you charged your phone. The energy didn't start at the plug socket – so where did it come from?

Your task: trace the energy back as far as you can. Write it as a chain of arrows, starting from the plug socket. Try for at least 4 steps.

Stuck? What's at the other end of the cables? What turns a generator?

📝 COPY INTO YOUR BOOK Most power stations use an energy resource to turn a turbine, which turns a generator to make electricity.

## Step 2: Read and write (12 minutes)

Copy these three questions into your book. Then read the text and answer them in full sentences.

- How do fossil fuel power stations make electricity?

- Give two problems with burning fossil fuels.

- Pick two renewable resources. For each, write one advantage and one disadvantage.

📖 READ Where does our electricity come from? Fossil fuels – coal, oil and gas – are burned in power stations. The energy released heats water to make steam. The steam turns a turbine, which turns a generator to make electricity. Fossil fuels are reliable, but burning them releases carbon dioxide, which contributes to climate change, and sulfur dioxide, which causes acid rain. They will also run out one day. Renewable resources won't run out. Wind turbines produce no carbon dioxide while generating, but only work when the wind blows and some people think they spoil the view. Solar cells produce no emissions, but give no electricity at night and less on cloudy days. Hydro-electric dams are reliable and can be switched on quickly, but flooding a valley destroys habitats. Tidal barrages are predictable, but change estuary habitats. Geothermal power stations use hot rocks underground, but only work in a few places.

📝 COPY AND COMPLETE Burning fossil fuels releases __________ __________, a greenhouse gas that contributes to __________ __________. It also releases __________ __________, which causes acid rain.

## Step 3: Sort it (5 minutes)

📝 COPY INTO YOUR BOOK Renewable resource: one that is being (or can be) replenished as it is used, so it will not run out. Non-renewable resource: one that will run out, because it is not replaced as fast as it is used.

Your task: draw two columns labelled Renewable and Non-renewable. Sort these: coal · wind · nuclear fuel · solar · oil · tidal · gas · hydro-electricity · bio-fuel · geothermal · water waves

Ask yourself: "Could we ever run out of this?"

## Step 4: Fact cards (10 minutes)

📝 COPY INTO YOUR BOOK Reliable: it can generate electricity whenever it is needed.

Your task: copy the table headings below and fill in one row for each of the 9 resources, using the fact cards.

| Resource | Renewable? | Reliable? | CO₂ when generating? | Other effects |
| --- | --- | --- | --- | --- |
| e.g. Wind | Yes | No – needs wind | None | Noisy, spoils view |

Fossil fuels (coal, oil, gas): burned to heat water; not renewable; reliable; release CO₂ (climate change) and sulfur dioxide (acid rain).

Nuclear fuel: fission heats water; not renewable; reliable; no CO₂ while generating; radioactive waste must be stored for a very long time; small risk of a major accident; expensive to build and shut down.

Wind: wind turns the blades; renewable; not reliable; no CO₂; noise and visual impact.

Solar: cells turn sunlight into electricity; renewable; not reliable (nothing at night); no CO₂; big farms use a lot of land.

Hydro-electricity: water from a dam flows through turbines; renewable; reliable and can be switched on very quickly; no CO₂; flooding valleys destroys habitats.

Tides: a barrage traps water at high tide; renewable; predictable; no CO₂; expensive and changes estuary habitats.

Water waves: up-and-down motion drives a generator; renewable; not reliable; no CO₂; can affect sea life and boats.

Geothermal: hot rocks heat water; renewable; reliable; very little CO₂; only works in a few places.

Bio-fuel: plant material or waste is burned; renewable; reliable; releases CO₂ but plants absorbed it while growing; uses land that could grow food.

## Step 5: Quick check (5 minutes)

Cover your table and answer from memory.

Q1 Name the three fossil fuels.

Q2 Why is nuclear fuel non-renewable?

Q3 Which renewable resource can be switched on quickly when demand suddenly rises?

Q4 Why is tidal power more reliable than wind power?

Q5 Give one problem that hydro-electric dams cause for the environment.

Q6 Why don't we use geothermal power everywhere?

📝 COPY AND COMPLETE Wind and solar power are unreliable because __________ and __________. Hydro-electricity is useful because it can be __________ __________ __________ when demand suddenly rises.

✅ Score out of 6. 4 or less? Re-read the fact cards for the ones you missed.

## Step 6: Main task – Power the Island (10 minutes)

You've been hired as the island's energy adviser! A small island gets all its electricity from diesel generators. The diesel is shipped in and costs £6 million a year. The island is very windy and sunny. The council has £80 million and three goals: 1. cut carbon dioxide emissions; 2. cut yearly running costs; 3. keep the lights on day and night.

| Option | Cost to build | Running cost per year | Reliable? | CO₂ | Other |
| --- | --- | --- | --- | --- | --- |
| Wind farm | £45 million | £1 million | No – depends on wind | None | Noise, visual impact, birds |
| Solar farm | £20 million | £0.5 million | No – none at night | None | Uses farmland |
| Tidal barrage | £90 million | £1 million | Predictable | None | Damages estuary habitat |
| Undersea cable to mainland | £60 million | £3 million | Yes | Depends on mainland | Could be damaged by storms |
| Keep diesel generators | £0 | £6 million if used all the time | Yes | High | Fuel must be shipped in |

📝 COPY INTO YOUR BOOK How to evaluate: FOR – at least 2 reasons, using data AGAINST – at least 2 problems JUDGEMENT – "Overall, the best choice is… because…"

Your task: write your advice to the council using the frame. Start with "My choice:" – which option(s) and the total cost (no more than £80 million).

Which option keeps the lights on when there's no wind and no sun?

## Step 7: Exam practice (11 minutes)

📝 COPY INTO YOUR BOOK COMPARE = talk about both options in every point. Use the word "whereas". EVALUATE = give points for and against, then a judgement.

Q1 (2 marks) Explain why a town cannot rely on a solar farm as its only supply of electricity.

Q2 (4 marks) A country could build a gas power station or an offshore wind farm, both with a maximum output of 800 MW. The gas station's average output is 90% of maximum, can generate whenever needed, must buy gas and releases CO₂. The wind farm's average output is 40% of maximum, only generates when windy, needs no fuel and releases no CO₂. Compare the two options.

Q3 (6 marks, Higher) Some people want to replace the UK's remaining fossil fuel power stations with nuclear power stations. Evaluate this plan.

✅ Add up your marks out of 12 (or out of 6 if you skipped Q3).

## Step 8: Exit ticket (2 minutes)

- The most important thing I learned today is…

- If I were in charge of the UK's electricity, I would use… because…

- One thing I'm still not sure about is…

# Answers

Only look at these once you have written your own answer!

### Step 1: Starter (5 minutes)

Plug socket ← National Grid cables ← generator at a power station ← turbine turned by steam, wind or water ← energy resource (e.g. gas burned to boil water, or the wind).

### Step 2: Read and write (12 minutes)

(1) The fuel is burned to heat water into steam, which turns a turbine connected to a generator.

(2) Any two: they release carbon dioxide (climate change); they release sulfur dioxide (acid rain); they will run out.

(3) e.g. wind – no fuel or emissions, but unreliable when the wind drops; solar – no emissions, but no output at night.

Gaps: carbon dioxide · climate change · sulfur dioxide

### Step 3: Sort it (5 minutes)

Non-renewable (4): coal, oil, gas, nuclear fuel. Renewable (7): wind, solar, tidal, hydro-electricity, bio-fuel, geothermal, water waves. Watch out – nuclear is NOT renewable.

### Step 5: Quick check (5 minutes)

Q1 Coal, oil and gas. Q2 The uranium is mined and will run out. Q3 Hydro-electricity. Q4 Tides happen at times we can predict; wind depends on the weather. Q5 Flooding a valley destroys habitats. Q6 It only works where hot rocks are near the surface.

Gaps: the wind doesn't always blow · there is no sunlight at night · switched on quickly

### Step 6: Main task – Power the Island (10 minutes)

Model: wind + solar (£65 million), keeping diesel as backup. FOR: no CO₂ when generating; running costs only £1.5 million a year instead of £6 million; the island is windy and sunny. AGAINST: neither is reliable, so diesel is still needed sometimes; noise, visual impact, uses farmland. JUDGEMENT: best choice – cuts CO₂ and costs the most within budget, and diesel backup keeps the lights on. (The tidal barrage is over budget.) Any choice is good if you use the data and finish with a clear judgement.

### Step 7: Exam practice (11 minutes)

Q1: Solar cells produce no electricity at night / less when cloudy (1), so there would be times with no supply – a reliable backup is needed (1).

Q2: Gas average output 720 MW whereas wind only 320 MW (1); gas generates whenever needed whereas wind only when windy (1); gas must buy non-renewable fuel whereas wind is free and renewable (1); gas releases CO₂ whereas wind releases none (1).

Q3: FOR – no CO₂ or sulfur dioxide when generating; reliable. AGAINST – non-renewable; radioactive waste stored for a very long time; small risk of accident; very expensive; slow to build and can't respond quickly to demand. JUDGEMENT – e.g. a good plan for reliable low-carbon electricity if combined with renewables.